🟦 1. Arduino Nano → MAX485 Wiring (Modbus RTU)

This is the RS-485 physical interface.
Everything here is mandatory for the device to behave as a proper Modbus slave.

Power

| MAX485 pin | Nano pin | Notes |
| --- | --- | --- |
| VCC | 5V | MAX485 runs on 5 V |
| GND | GND | Must share the same ground |

Serial + Direction Control

| MAX485 pin | Nano pin | Purpose |
| --- | --- | --- |
| RO | D0 (RX) | RS-485 bus → Nano |
| DI | D1 (TX) | Nano → RS-485 bus |
| RE | D2 | Tie RE + DE together |
| DE | D2 | D2 controls transmit/receive |

Important:
RE and DE must be tied together:

MAX485 RE ┐

├── Nano D2

MAX485 DE ┘

RS-485 twisted-pair bus

This connects to your USB-RS485 adapter or multi-device bus.

| MAX485 pin | RS-485 network |
| --- | --- |
| A | A / D+ / + |
| B | B / D− / − |

🟩 2. Arduino Nano → AS7341 spectral sensor (I²C)

The DFRobot AS7341 breakout is 5 V-safe.

Power

| AS7341 pin | Nano pin |
| --- | --- |
| + | 5V |
| - | GND |

I²C

| AS7341 pin | Nano pin |
| --- | --- |
| SCL | A5 |
| SDA | A4 |

Unused pins

| AS7341 pin | Connection |
| --- | --- |
| INT | Not connected |
| GPIO | Not connected |

🟧 3. Arduino Nano → LED Module (DFRobot LED or similar)

Your LED control pin is D4.

Wiring

| Component | Nano pin | Notes |
| --- | --- | --- |
| LED module signal | D4 | Controlled by Modbus register 40001 |
| LED module VCC | 5V | Depending on module |
| LED module GND | GND | Common ground mandatory |

Output logic (in code):

- 0 → LED OFF

- Anything else → LED ON

🟨 4. Arduino Nano → Relay Module (on D6)

Most typical Arduino relay boards have:

- VCC (5 V)

- GND

- IN (signal input)

Wiring

| Relay pin | Nano pin |
| --- | --- |
| IN | D6 |
| VCC | 5V |
| GND | GND |

Relay logic (as implemented):

- 0 → relay OFF

- Any non-zero value → relay ON

If you have an active-LOW relay, tell me and I’ll invert the logic.

🟪 5. Final Register Map (Exactly Matching Current Code)

This is the complete authoritative map for Modbus register reads/writes.

Registers start at address 0 (Modbus 40001)

Type: Holding Registers (FC 0x03 / 0x06)

| Index | Modbus Address | 400xx | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 0 | 40001 | REG_LED_CONTROL | 0 = LED OFF (D4), ≠0 = LED ON |
| 1 | 1 | 40002 | REG_RELAY_CONTROL | 0 = Relay OFF (D6), ≠0 = Relay ON |
| 2 | 2 | 40003 | REG_AS7341_F1 | Spectral channel F1 |
| 3 | 3 | 40004 | REG_AS7341_F2 | Spectral channel F2 |
| 4 | 4 | 40005 | REG_AS7341_F3 | Spectral channel F3 |
| 5 | 5 | 40006 | REG_AS7341_F4 | Spectral channel F4 |
| 6 | 6 | 40007 | REG_AS7341_F5 | Spectral channel F5 |
| 7 | 7 | 40008 | REG_AS7341_F6 | Spectral channel F6 |
| 8 | 8 | 40009 | REG_AS7341_F7 | Spectral channel F7 |
| 9 | 9 | 40010 | REG_AS7341_F8 | Spectral channel F8 |
| 10 | 10 | 40011 | REG_AS7341_CLEAR | CLEAR channel |
| 11 | 11 | 40012 | REG_AS7341_NIR | NIR channel |
| 12 | 12 | 40013 | REG_AS7341_STATUS_WORD | 0=OK, 1=init fail, 2=read error |